# WP 5: Development of an automated data fusion with edge fitting - code documentation

## Overview

- Direct data comparison between newly captured UAV and existing ALS data

- Automatic stable area detection

- ICP: algorithm to minimize point distance between UAV and ALS

- Fusion of ALS and UAV with edge fitting

## Direct data comparison

Script for a quality check:

Input: Surface UAV data and Surface ALS data (with roofs etc.)

- Direct_datacomparison.py

Output: gives 3 histograms of difference in x, y, and z between the point clouds. Z will be the least precise, since it still includes changed areas (newly built buildings, vegetation). First indication of quality.

## Automatic stable area detection

Script for automatic detecting stable areas, cutting out the changed areas of the ALS data to leave them out in the fusion process.

Input: Surface ALS and UAV

- Stablearea_detection.py

Output: two pointclouds, changed and stable

## ICP

Script for minimizing point distance between:

Input: stable area ALS (output 2. – classified roof and ground) and classification WP4: only roofs and streets

- ICP_simple.py

Output: transformation matrix and RMSE

- Apply transformation matrix on classified ground pointcloud of UAV (output of WP4)

## Data fusion with edge fitting

Input: Ground ALS and Transformed ground UAV (output of 3.)

- Fusion_gaussianautomatic.py

Output: fused point cloud with edge fitting

- Explore_edge.py  edge distribution heights and can flag edges that are higher than custom put in value (H_max).